City of Flint - Water Pollution Control

General Concrete Installation and Repair Specifications

1.0	General.

1.1	Description:  The work specified herein consists of furnishing the equipment, labor, materials, and testing required for the construction or use of Portland Cement, cast-in-place, and preset concrete items.  All work shall conform to all local building codes.

1.2	Reference Documents.

1.2.1	American Concrete Institute (ACI)

1.2.2	American Society for Testing and Materials (ASTM)

1.2.3	Portland Cement Association (PCA)

2.0.	Warranty.

2.1	The Contractor shall expressly represent, warrant and agree that all goods, equipment, material, services or work provided or performed on or off City premises relative to this work shall:

2.1.1	Be free of defects in workmanship and materials for a period of twelve (12) months after the final acceptance of the project by the City.  All cost of replacement parts and labor required to repair any defects during the guarantee period are considered to be included in the bid price for the equipment.

2.1.2	Conform in all respects to the City’s specifications, requirements or other descriptions.

2.1.3	Be fit for the purpose(s) of intended use.

2.1.4	Be new and good quality, merchantable, of good workmanship, of materials best suited for the intended purposes, and free from defects of any kind.

2.2	Any usage statement, guarantee, or warranty stated in the Bid relating to equipment, materials, service, or Work will be superseded by the terms and conditions stated herein or in their absence by the following:

2.2.1	If said equipment, material, service or Work is found to be defective in workmanship and materials, design fails, or is found to be nonconforming with the terms and conditions of this Contract with the time specified in paragraph 2.1 of this section, it shall, at the City’s option, be repaired or replaced at the City’s location by the Contractor at absolutely no cost to the City.

2.2.2	All items delivered or supplied hereunder will be free and clear of all liens, encumbrances, claims, and security interests of whatever nature and substance.  Upon request, the Contractor will supply the City with documentation satisfactory to it evidencing the absence of such liens, encumbrances, claims, and security interests.

CITY OF FLINT - WATER POLLUTION CONTROL

CONCRETE AND CONCRETE INSTALLATION

BID WORKSHEET – (2) Years

| Quantity | Item | Price per Yard | Extended Price |
| --- | --- | --- | --- |
| 75 yd3 | Class 3000 psi concrete installed – Including rebar, dowels, wire mesh installed and troweled. Prep work such as, but not limited to, building forms, using a bonding agent, leveling and compacting soil, and cleanup of area once complete. |  |  |
| 75 yd3 | Class 4000 psi concrete installed – Including rebar, dowels, wire mesh installed and troweled. Prep work such as, but not limited to, building forms, using a bonding agent, leveling and compacting soil, and cleanup of area once complete. |  |  |
| 75 yd3 | Class 7000 psi concrete installed – Including rebar, dowels, wire mesh installed and troweled. Prep work such as, but not limited to, building forms, using a bonding agent, leveling and compacting soil, and cleanup of area once complete. |  |  |
| 1 | Concrete and labor to install a class 3000 psi concrete pump pad 31” X 71.5” - 17” Tall. All material and labor to be included. |  |  |
| 1 | Grout and labor to install a pad 3’ X 6’ X 2” Tall. |  |  |
| Quantity | Item | Price per FT | Extended Price |
| 10 ft | Clean and high pressure inject structural cracks with Emecole 121 crack 1:1 mv |  |  |
| 10 ft | The bid shall include the cost of repairs of the necessary expansion joints. Ceiling joint material shall be "combiflex" material and "Jenny" joint material used for sidewalls and floor repairs or an acceptable alternative |  |  |
| 200 ft | The bid shall include cost of repairs for leaks to include - port, seal and inject with a flexible polyurethane foam, AZO grout 424. |  |  |
| Quantity | Item | Price per Hour | Extended Price |
| 1 | Labor Rate |  |  |
| 1 | Labor Rate – Overtime/Saturday/Holiday |  |  |
| Independent Testing Laboratory |  |  |  |
| Item |  | % MARK-UP |  |
| Percent mark-up on all parts and subcontracted labor (if any) |  |  |  |
| Bid Total Per Year |  | $ |  |

All proposals will be evaluated on the following criteria:

1. Failure to use this bid form shall result in bid disqualification.

2. Failure to bid on all items shall result in an "incomplete bid" determination.

3. List value-added considerations on a separate sheet of paper.

Terms:					Dest:				Fed. ID:

Firm Name:

Address:							City/State/Zip

Phone:			Fax:				Email/Website:

Signed:							Date:

Please submit one (1) original and one (1) copy.

Bid results may be viewed next business day online at www.cityofflint.com/purchasing